| Ares-P12A Ares-P9A | 39CH Mode1 | 34CH Mode2 | DMX Value | Function |
| --- | --- | --- | --- | --- |
| Dimmer | 1 | 1 | 0---255 | Brightness--->Darkness |
| Dimmer FINE | 2 |  | 0---255 |  |
| Strobe | 3 | 2 | 0---3 4---103 104---107 108---207 208---212 213---225 226---238 239---251 252---255 | Closed Slow Strobe--->Fast Strobe Open SlowPuls---->Fast Puls Open Random Slow Strobe Random Medium Strobe Random Fast Strobe Open |
| Color | 4 | 3 | 0---11 12---23 24--- 35 36--- 47 48--- 59 60---71 72--- 83 84---95 96---107 108---119 120--- 131 132---143 144---155 156--- 171 172--- 213 214--- 255 | White Color1+White Color1 Color1 + Color2 Color2 Color2 + Color3 Color3 Color3 + Color4 Color4 Color4 + Color5 Color5 Color5 + Color6 Color6 Color6 + White CW Rotation Slow--->Fast CCW Rotation Fast--->Slow |
| Cyan | 5 | 4 | 0---255 | EXCLUDED---->INSERTED |
| Magenta | 6 | 5 | 0---255 | EXCLUDED---->INSERTED |
| Yellow | 7 | 6 | 0---255 | EXCLUDED---->INSERTED |
| CTC | 8 | 7 | 0---255 | EXCLUDED---->INSERTED |
| Macro Color | 9 |  | 0---255 |  |
| IRIS | 10 | 8 | 0---128 129---131 132---171 172---211 212---251 252---255 | MAX----->MIN Close Slow Puls---->Fast Puls Slow Puls Fast Close---->Fast Puls Fast Close Slow Puls Fast Open---->Fast Puls Fast Open Open |
| Gobo1 Select | 11 | 9 | 0---9 10--- 20 21--- 29 30---39 40--- 49 50--- 59 60---85 86---111 112---137 138---163 164---189 190---222 223---255 | White Gobo1 Gobo2 Gobo3 Gobo4 Gobo5 Gobo1 Shake Slow--->Fast Gobo2 Shake Slow--->Fast Gobo3 Shake Slow--->Fast Gobo4 Shake Slow--->Fast Gobo5 Shake Slow--->Fast CW Rotation Slow---->Fast CCW Rotation Fast--->Slow |
| Gobo1 Index Rotation | 12 | 10 | 0---127 128---191 192---255 | 0°---->360°(FINE IN 13CH) CW Rotating Slow---Fast CCW Rotation Fast---Slow |
| Gobo1 Index Rotation FINE | 13 |  | 0---255 |  |
| Fixed Gobo | 14 | 11 | 0---4 5---10 11---16 17---22 23---28 29---34 35---40 41---46 47---52 53---58 59---74 75---90 91---106 107---122 123---138 139---154 155---170 171--- 186 187--- 202 203---229 230---255 | White Gobo1 Gobo2 Gobo3 Gobo4 Gobo5 Undefined Undefined Animation Animation Gobo1Shake Slow--->-Fast Gobo2 Shake Slow--->-Fast Gobo3 Shake Slow---->Fast Gobo4 Shake Slow---->Fast Gobo5Shake Slow---->Fast Gobo6 Shake Slow---->Fast Gobo7 Shake Slow---->Fast Gobo8 Shake Slow---->Fast Gobo9 Shake Slow---->Fast CW Rotation Slow--->Fast CCW Rotation Fast--->Slow |
| BLADE1 | 15 | 12 | 0---255 | EXCLUDED---->INSERTED |
| BLADE2 | 16 | 13 | 0---255 | EXCLUDED---->INSERTED |
| BLADE3 | 17 | 14 | 0---255 | EXCLUDED---->INSERTED |
| BLADE4 | 18 | 15 | 0---255 | EXCLUDED---->INSERTED |
| BLADE5 | 19 | 16 | 0---255 | EXCLUDED---->INSERTED |
| BLADE6 | 20 | 17 | 0---255 | EXCLUDED---->INSERTED |
| BLADE7 | 21 | 18 | 0---255 | EXCLUDED---->INSERTED |
| BLADE8 | 22 | 19 | 0---255 | EXCLUDED---->INSERTED |
| FRAMING ROTATION | 23 | 20 | 0---255 | Position 0°---->90° |
| Undefined | 24 | 21 | 0---10 11----133 134---255 | Undefined |
| Prism1 | 25 | 22 | 0---127 128---255 | EXCLUDED INSERTED |
| PrismRotation1 | 26 | 23 | 0---127 128---190 191---194 195---255 | Position 0°-->360° CW Rotation Fast---> Slow STOP CCW Rotation Slow--->Fast |
| Prism2 | 27 | 24 | 0---127 128---255 | EXCLUDED INSERTED |
| PrismRotation2 | 28 | 25 | 0---127 128---190 191---194 195---255 | Position 0°-->360° CW Rotation Fast---> Slow STOP CCW Rotation Slow--->Fast |
| Focus | 29 | 26 | 0---255 |  |
| Focus FINE | 30 |  | 0---255 |  |
| Zoom | 31 | 27 | 0---255 |  |
| Zoom FINE | 32 |  | 0---255 |  |
| PAN(X) | 33 | 28 | 0---255 |  |
| PAN(X) FINE | 34 | 29 | 0---255 |  |
| TILTE(Y) | 35 | 30 | 0---255 |  |
| TILTE(Y) FINE | 36 | 31 | 0---255 |  |
| P/T SPEED | 37 | 32 | 0---128 129---170 171---211 212---255 | Undefined Undefined Undefined Undefined |
| Lamp Control | 38 | 33 | 0---32 33---64 65---96 97---128 129---160 161---193 194---225 226---255 | Undefined Undefined Undefined Undefined Undefined Undefined Undefined Undefined |
| Control | 39 | 34 | 0---127 128---153 154---179 180---205 206---231 232---255 | Undefined Undefined Undefined Undefined Undefined Reset ALL( > 3s) |